# GridVision 无人机巡检系统 — 软著提交核心源代码（Clean版）

本文档为 GridVision 系统核心源代码净化版，已移除调试日志、审计统计与专项排查代码，保留巡检点生成、任务规划、路径规划、重规划、Dashboard 导出、地图渲染、巡检播放与图传展示等核心业务逻辑。

# 净化统计

console.log: 9
console.warn: 6
console.error: 0
print: 65
audit函数: 3
audit调用: 5
debug注释: 0
Clean前页数(估): 44
Clean后页数(估): 40

# backend/01_point_generation.py

"""

GridVision 毕设答辩核心代码摘录 — 巡检点生成

源文件: core/inspection_point_generator.py

行号: 166-313, 530-621

含沿折线采样、图像检测点吸附到拓扑。

本文件为摘录，非可独立运行模块；完整依赖见原工程。

"""

# ===== inspection_point_generator.py L166-L313 =====

def generate_line_inspection_points(

line,  # IndependentLine 对象

terrain: Optional[np.ndarray] = None,

flight_height: float = 25.0,

spacing: float = 100.0,

angle_threshold_deg: float = 25.0,

max_points_per_line: int = 50

) -> List[LineInspectionPoint]:

"""

对单条独立线路生成巡检点

规则：

- 起点、终点必设（endpoint，high priority）

- 明显拐点必设（turning，high priority）

- 长线段等距补点（sample，normal priority）

- 限制每条线最大点数，避免过密

Args:

line: IndependentLine 对象

terrain: 地形高度图（可选）

flight_height: 飞行高度（米）

spacing: 采样间距（像素）

angle_threshold_deg: 拐点角度阈值

max_points_per_line: 每条线最大点数限制

Returns:

List[LineInspectionPoint]: 巡检点列表

"""

if not line.ordered_pixels:

return []

polyline = line.ordered_pixels

points = []

point_count = 0

used_indices = set()  # 记录已使用的索引，避免重复点

# 1. 端点（起点和终点）- 最高优先级

if len(polyline) >= 2:

# 起点

start_pos = polyline[0]

start_pos_3d = _map_to_3d(start_pos, terrain, flight_height)

points.append(LineInspectionPoint(

id=f"{line.id}_P_{point_count:03d}",

line_id=line.id,

point_type="endpoint",

pixel_position=(float(start_pos[0]), float(start_pos[1])),

position_3d=start_pos_3d,

line_index=0,

priority="high",

status="uninspected",

source_reason="start_endpoint"

))

used_indices.add(0)

point_count += 1

# 终点

end_pos = polyline[-1]

end_pos_3d = _map_to_3d(end_pos, terrain, flight_height)

points.append(LineInspectionPoint(

id=f"{line.id}_P_{point_count:03d}",

line_id=line.id,

point_type="endpoint",

pixel_position=(float(end_pos[0]), float(end_pos[1])),

position_3d=end_pos_3d,

line_index=len(polyline) - 1,

priority="high",

status="uninspected",

source_reason="end_endpoint"

))

used_indices.add(len(polyline) - 1)

point_count += 1

# 2. 拐点检测 - 高优先级，确保保留

turning_indices = detect_turning_points(polyline, angle_threshold_deg)

added_turning = 0

for idx in turning_indices:

# 跳过起点和终点附近的拐点

if idx < 5 or idx > len(polyline) - 5:

continue

# 避免重复添加

if idx in used_indices:

continue

pos = polyline[idx]

pos_3d = _map_to_3d(pos, terrain, flight_height)

points.append(LineInspectionPoint(

id=f"{line.id}_P_{point_count:03d}",

line_id=line.id,

point_type="turning",

pixel_position=(float(pos[0]), float(pos[1])),

position_3d=pos_3d,

line_index=idx,

priority="high",

status="uninspected",

source_reason=f"turning_point_angle_{angle_threshold_deg}"

))

used_indices.add(idx)

added_turning += 1

point_count += 1

# 3. 等距采样点 - 普通优先级，受最大点数限制

remaining_quota = max_points_per_line - len(points)

samples = sample_points_along_polyline(polyline, spacing)

added_samples = 0

for idx, dist, pos in samples:

# 达到最大点数限制

if added_samples >= remaining_quota:

break

# 避免与端点重复

if idx < 5 or idx > len(polyline) - 5:

continue

# 避免与拐点重复

if idx in used_indices:

continue

# 避免与已有采样点位置重复

is_duplicate = any(abs(pos[0] - p.pixel_position[0]) < 3 and

abs(pos[1] - p.pixel_position[1]) < 3

for p in points if p.point_type == "sample")

if is_duplicate:

continue

pos_3d = _map_to_3d(pos, terrain, flight_height)

points.append(LineInspectionPoint(

id=f"{line.id}_P_{point_count:03d}",

line_id=line.id,

point_type="sample",

pixel_position=pos,

position_3d=pos_3d,

line_index=idx,

priority="normal",

status="uninspected",

source_reason=f"sample_spacing_{spacing}px"

))

used_indices.add(idx)

added_samples += 1

point_count += 1

# 按线路索引排序

points.sort(key=lambda p: p.line_index)

return points

# ===== inspection_point_generator.py L530-L621 =====

def build_image_detected_inspection_points(

detections: List[Dict[str, Any]],

topo_graph,

*,

terrain: Optional[np.ndarray] = None,

flight_height: float = 25.0,

max_snap_distance: float = 30.0,

) -> Tuple[List[LineInspectionPoint], Dict[str, List[LineInspectionPoint]], List[Dict[str, Any]]]:

"""Convert detected image points into snapped LineInspectionPoint objects."""

from core.topo_task import snap_point_to_topo_graph

points_by_line: Dict[str, List[LineInspectionPoint]] = {}

overlay: List[Dict[str, Any]] = []

all_points: List[LineInspectionPoint] = []

invalid_count = 0

for det in detections or []:

raw = det.get("coord") or det.get("pixel_position")

if not raw or len(raw) < 2:

continue

raw_xy = (float(raw[0]), float(raw[1]))

snap_strict = snap_point_to_topo_graph(raw_xy, topo_graph, max_distance=max_snap_distance)

snap_any = snap_point_to_topo_graph(raw_xy, topo_graph, max_distance=None)

snap = snap_strict or snap_any

entry = {

"id": det.get("id"),

"raw_coord": [round(raw_xy[0], 2), round(raw_xy[1], 2)],

"valid": snap is not None,

"confidence": det.get("confidence"),

"source": det.get("source"),

}

if snap is None:

invalid_count += 1

entry["snap_distance"] = None

overlay.append(entry)

continue

snap_fallback = snap_strict is None

dist_px = float(snap["distance"])

snapped = snap["snapped_coord"]

pos_3d = _map_to_3d(raw_xy, terrain, flight_height)

point = LineInspectionPoint(

id=str(det.get("id") or f"IP_{len(all_points) + 1:04d}"),

line_id=str(snap["line_id"]),

point_type="image_detected",

pixel_position=(float(raw_xy[0]), float(raw_xy[1])),

position_3d=pos_3d,

line_index=int(snap.get("segment_index", 0)),

priority="high",

status="uninspected",

source_reason="image_snap_fallback" if snap_fallback else "image_black_dot",

detection_result={

"raw_coord": [round(raw_xy[0], 2), round(raw_xy[1], 2)],

"edge_id": snap["edge_id"],

"distance_along_edge": round(float(snap["distance_along_edge"]), 2),

"snap_distance": round(dist_px, 2),

"snap_distance_px": round(dist_px, 2),

"snapped_coord": [round(float(snapped[0]), 2), round(float(snapped[1]), 2)],

"confidence": det.get("confidence"),

"source": det.get("source"),

"snap_fallback": snap_fallback,

},

)

points_by_line.setdefault(point.line_id, []).append(point)

all_points.append(point)

entry.update(

{

"edge_id": snap["edge_id"],

"snapped_coord": [round(float(snapped[0]), 2), round(float(snapped[1]), 2)],

"distance_along_edge": round(float(snap["distance_along_edge"]), 2),

"snap_distance": round(dist_px, 2),

"snap_fallback": snap_fallback,

}

)

overlay.append(entry)

for line_id in points_by_line:

points_by_line[line_id].sort(

key=lambda p: float((p.detection_result or {}).get("distance_along_edge", p.line_index))

)

return all_points, points_by_line, overlay

# backend/02_mission_generation.py

"""

GridVision 毕设答辩核心代码摘录 — 巡检任务生成

源文件: planner/powerline_planner_v3_final.py + core/topo_task.py

行号: 2364-2510, 644-722

PowerlinePlannerV3 巡检点步骤 + build_edge_tasks。

本文件为摘录，非可独立运行模块；完整依赖见原工程。

"""

# ===== powerline_planner_v3_final.py L2364-L2510 =====

def step5_generate_line_inspection_points(

self,

spacing=100.0,

angle_threshold_deg=25.0,

max_points_per_line=50

):

"""

STEP 5 (新): 为多条独立线路生成巡检点

规则：

- 起点、终点必设（endpoint，high priority）

- 明显拐点必设（turning，high priority）

- 长线段等距补点（sample，normal priority）

- 限制每条线最大点数，避免过密

Args:

spacing: 采样间距（像素）

angle_threshold_deg: 拐点角度阈值

max_points_per_line: 每条线最大点数限制

Returns:

List[LineInspectionPoint]: 所有巡检点

"""

if not self.independent_lines:

return []

from core.inspection_point_generator import (

generate_all_inspection_points,

save_inspection_points_visualization,

get_inspection_points_statistics

)

# 使用当前地形（如果可用）

terrain = self.height_map_smooth if hasattr(self, 'height_map_smooth') else None

# 生成巡检点

self.line_inspection_points, self.line_inspection_points_by_line = (

generate_all_inspection_points(

self.independent_lines,

terrain=terrain,

flight_height=self.flight_height,

spacing=spacing,

angle_threshold_deg=angle_threshold_deg,

max_points_per_line=max_points_per_line

)

)

# 保存可视化

save_inspection_points_visualization(

self.independent_lines,

self.line_inspection_points_by_line,

self.image_path,

"result/step5_line_inspection_points.png"

)

# 打印统计信息

stats = get_inspection_points_statistics(

self.line_inspection_points,

self.line_inspection_points_by_line

)

return self.line_inspection_points

def load_real_satellite_manual_annotations(self, manual_json_path: str):

"""从手工标注 JSON 加载线路与巡检点（跳过 CV 检测）。"""

from core.real_satellite_manual import apply_manual_dataset_to_planner

return apply_manual_dataset_to_planner(self, manual_json_path)

def step5_detect_image_inspection_points(self, detector_config=None):

"""Detect inspection points from the input image (green circles on real map)."""

from core.inspection_point_detector import (

detect_black_inspection_points_with_stats,

generate_clean_map,

resolve_detector_config,

)

cfg = resolve_detector_config(self.image_path, detector_config)

self.inspection_detector_config = cfg

red_mask = self.mask if getattr(self, "mask", None) is not None else None

self.image_inspection_detections, self.image_detection_stats = (

detect_black_inspection_points_with_stats(

self.image_path, config=cfg, red_mask=red_mask

)

)

try:

from core.real_map_cv import (

extract_real_map_green_mask,

is_real_map_detection_image,

save_green_mask,

)

if is_real_map_detection_image(self.image_path):

rgb_u8 = np.array(self.image.convert("RGB"))

green_mask = extract_real_map_green_mask(rgb_u8, red_mask=red_mask)

save_green_mask(green_mask)

except Exception as exc:

try:

from core.real_map_cv import is_real_map_detection_image

real_map_same_image = is_real_map_detection_image(self.image_path)

except ImportError:

real_map_same_image = False

if real_map_same_image:

self.clean_map_path = str(self.image_path).replace("\\", "/")

elif os.environ.get("UAV_DISPLAY_MAP", "").strip() and os.path.isfile(

os.environ.get("UAV_DISPLAY_MAP", "").strip()

):

display_map = os.environ.get("UAV_DISPLAY_MAP", "").strip()

self.clean_map_path = display_map.replace("\\", "/")

else:

try:

self.clean_map_path = generate_clean_map(

self.image_path,

detections=self.image_inspection_detections,

config=cfg,

)

except Exception as exc:

self.clean_map_path = None

self.inspection_point_source = "image"

self.line_inspection_points = []

self.line_inspection_points_by_line = {}

label = "绿色巡检圆" if self.image_detection_stats.get("detector") == "green_hsv" else "图像特征点"

return self.image_inspection_detections

# ===== topo_task.py L644-L722 =====

def build_edge_tasks(topo_graph, line_inspection_points_by_line: Dict[str, List]) -> List[EdgeTask]:

"""

按 line_id 连通链构建合并 EdgeTask（每链一个任务，仅包含有巡检点的链）。

"""

line_ids = sorted({e.line_id for e in topo_graph.edges.values()})

chain_specs_by_line = {

lid: _build_line_chain_specs(topo_graph, lid, log_merge=True) for lid in line_ids

}

edge_points = map_points_to_edges(

topo_graph, line_inspection_points_by_line, chain_specs_by_line=chain_specs_by_line

)

edge_tasks: List[EdgeTask] = []

merged_topo_count = 0

lines_with_tasks: Set[str] = set()

for line_id in line_ids:

for cid, ordered, merged, uu, vv in chain_specs_by_line.get(line_id, []):

pts = edge_points.get(cid, [])

if not pts:

continue

pts_sorted = sorted(

pts,

key=lambda p: float(

p.get("distance_along_edge", 0.0)

if isinstance(p, dict)

else getattr(p, "distance_along_edge", 0.0)

),

)

merged_len = _polyline_len2d(merged)

straight = all(

(topo_graph.edges[eid].is_straight for eid in ordered if eid in topo_graph.edges)

) if ordered else True

task = EdgeTask(

edge_id=cid,

u=uu or (topo_graph.edges[ordered[0]].u if ordered else ""),

v=vv or (topo_graph.edges[ordered[-1]].v if ordered else ""),

line_id=line_id,

polyline=list(merged),

pixel_polyline=list(merged),

original_polyline=list(merged),

image_polyline=list(merged),

len2d=float(merged_len),

inspection_points=pts_sorted,

num_points=len(pts_sorted),

is_straight=bool(straight),

split_reason="merged_chain",

meta={"chain_topo_edge_ids": list(ordered)},

)

edge_tasks.append(task)

merged_topo_count += len(ordered)

lines_with_tasks.add(line_id)

fallback_count = sum(

1 for p in pts_sorted if _point_mapping_mode(p).endswith("_fallback")

)

total_topo = len(topo_graph.edges)

edge_task_pts = sum(len(t.inspection_points or []) for t in edge_tasks)

return edge_tasks

# backend/03_path_planning.py

"""

GridVision 毕设答辩核心代码摘录 — 拓扑路径规划

源文件: planner/topo_dijkstra.py

行号: 98-165, 303-400

Dijkstra 最短路径与连接段生成。

本文件为摘录，非可独立运行模块；完整依赖见原工程。

"""

# ===== topo_dijkstra.py L98-L165 =====

def dijkstra_shortest_path(

topo_graph: TopoGraph,

start_node_id: str,

goal_node_id: str,

cost_config: Optional[Dict[str, Any]] = None,

) -> Optional[Dict[str, Any]]:

"""

节点间最小总代价路径（Dijkstra）。

Returns:

{"node_path", "edge_path", "total_cost"} 或 None（不可达）

"""

if start_node_id not in topo_graph.nodes or goal_node_id not in topo_graph.nodes:

return None

if start_node_id == goal_node_id:

return {

"node_path": [start_node_id],

"edge_path": [],

"total_cost": 0.0,

}

adj = build_weighted_node_adjacency(topo_graph, cost_config)

dist: Dict[str, float] = {start_node_id: 0.0}

prev_node: Dict[str, Optional[str]] = {start_node_id: None}

prev_edge: Dict[str, Optional[str]] = {start_node_id: None}

heap: List[Tuple[float, str]] = [(0.0, start_node_id)]

settled: set = set()

while heap:

d, u = heapq.heappop(heap)

if u in settled:

continue

settled.add(u)

if u == goal_node_id:

break

for v, eid, w in adj.get(u, []):

nd = d + w

if nd < dist.get(v, float("inf")):

dist[v] = nd

prev_node[v] = u

prev_edge[v] = eid

heapq.heappush(heap, (nd, v))

if goal_node_id not in dist:

return None

node_path: List[str] = []

edge_path: List[str] = []

cur = goal_node_id

while cur is not None:

node_path.append(cur)

eid = prev_edge.get(cur)

if eid and cur != start_node_id:

edge_path.append(eid)

cur = prev_node.get(cur)

node_path.reverse()

edge_path.reverse()

return {

"node_path": node_path,

"edge_path": edge_path,

"total_cost": float(dist[goal_node_id]),

}

# ===== topo_dijkstra.py L303-L400 =====

def generate_connection_segment_with_planner(

point_a: Tuple[float, float],

point_b: Tuple[float, float],

topo_graph: TopoGraph,

edge_task_map: dict,

connect_planner: str = "bfs",

cost_config: Optional[Dict[str, Any]] = None,

use_proximity_bfs: bool = True,

from_edge_id: Optional[str] = None,

to_edge_id: Optional[str] = None,

) -> Tuple[List[Tuple[float, float]], float]:

"""

沿拓扑图生成连接段；connect_planner: 'bfs' | 'dijkstra'。

Dijkstra 失败时 fallback 到 BFS。

"""

def _append_polyline(dst: List[Tuple[float, float]], src: List[Tuple[float, float]]) -> None:

if not src:

return

src2 = [(float(p[0]), float(p[1])) for p in src]

if not dst:

dst.extend(src2)

return

if np.linalg.norm(np.array(dst[-1]) - np.array(src2[0])) <= 1e-6:

dst.extend(src2[1:])

else:

dst.extend(src2)

def _geom_length(geom: List[Tuple[float, float]]) -> float:

if len(geom) < 2:

return 0.0

return float(sum(np.linalg.norm(np.array(geom[i + 1]) - np.array(geom[i])) for i in range(len(geom) - 1)))

def _point_to_anchor(edge_task, point_xy: Tuple[float, float], anchor_node: str):

poly = list(getattr(edge_task, "polyline", None) or [])

if len(poly) < 2:

return None

s = _project_point_to_polyline_distance(point_xy, poly)

if s is None:

return None

total = _polyline_length(poly)

s = max(0.0, min(float(s), total))

if anchor_node == edge_task.u:

geom = _slice_polyline_by_distance(poly, 0.0, s)

return {"cost": s, "point_to_anchor": list(reversed(geom))}

if anchor_node == edge_task.v:

geom = _slice_polyline_by_distance(poly, s, total)

return {"cost": total - s, "point_to_anchor": geom}

return None

def _anchor_to_point(edge_task, point_xy: Tuple[float, float], anchor_node: str):

info = _point_to_anchor(edge_task, point_xy, anchor_node)

if not info:

return None

return list(reversed(info["point_to_anchor"]))

from_edge = edge_task_map.get(from_edge_id) if from_edge_id else None

to_edge = edge_task_map.get(to_edge_id) if to_edge_id else None

# 同边直接切片（target-aware）

if from_edge is not None and to_edge is not None and from_edge.edge_id == to_edge.edge_id:

poly = list(from_edge.polyline or [])

if len(poly) >= 2:

sa = _project_point_to_polyline_distance(point_a, poly)

sb = _project_point_to_polyline_distance(point_b, poly)

if sa is not None and sb is not None:

sliced = _slice_polyline_by_distance(poly, sa, sb)

return sliced, _polyline_length(sliced)

# 端点锚点组合（dijkstra/bfs 按 connect_planner 选）

if from_edge is not None and to_edge is not None:

best = None

for a_anchor in (from_edge.u, from_edge.v):

a_info = _point_to_anchor(from_edge, point_a, a_anchor)

if not a_info:

continue

for b_anchor in (to_edge.u, to_edge.v):

b_info = _point_to_anchor(to_edge, point_b, b_anchor)

if not b_info:

continue

if a_anchor == b_anchor:

node_path = [a_anchor]

middle_cost = 0.0

elif connect_planner.lower() == "dijkstra":

rs = dijkstra_shortest_path(topo_graph, a_anchor, b_anchor, cost_config=cost_config)

node_path = (rs or {}).get("node_path") or []

middle_cost = float((rs or {}).get("total_cost", float("inf")))

else:

node_path = get_shortest_path(topo_graph, a_anchor, b_anchor) or []

middle_cost = estimate_bfs_path_cost(topo_graph, node_path) if node_path else float("inf")

if not node_path:

continue

middle_geo, _ = expand_node_path_to_geometry(

topo_graph.get_node(node_path[0]).pos2d,

topo_graph.get_node(node_path[-1]).pos2d,

# backend/04_replanning.py

"""

GridVision 毕设答辩核心代码摘录 — 起终点重规划

源文件: planner/replan_start_end.py

行号: 1401-1705

build_start_end_replan_mission 完整摘录。

本文件为摘录，非可独立运行模块；完整依赖见原工程。

"""

# ===== replan_start_end.py L1401-L1705 =====

def build_start_end_replan_mission(

base_mission: Dict[str, Any],

start_xy: List[float],

end_xy: List[float],

planning_spacing: float = 70.0,

weather_context: Optional[Dict[str, Any]] = None,

mission_context: Optional[Dict[str, Any]] = None,

) -> Dict[str, Any]:

"""

Build full mission JSON: start → inspect all edges (dense polylines) → end.

"""

base_mission = copy.deepcopy(base_mission)

_merge_mission_metadata_nested(base_mission)

iw, ih = replan_image_bounds_from_baseline(base_mission)

start, end = validate_image_coords(start_xy, end_xy, width=iw, height=ih)

spacing = max(20.0, float(planning_spacing))

ctx_pre = mission_context or {}

base_meta_early = base_mission.get("metadata") or {}

image_rel = str(

ctx_pre.get("image_path")

or base_meta_early.get("map_image")

or "data/test.png"

).replace("\\", "/")

src_for_topo = normalize_inspection_point_source(

ctx_pre.get("inspection_point_source")

or base_meta_early.get("inspection_point_source")

or "spacing"

)

topo_graph, edge_task_map = _try_load_topo_for_replan(

_project_root(),

image_rel,

src_for_topo,

spacing,

)

tasks = _extract_inspect_tasks(base_mission, edge_task_map=edge_task_map or {})

if edge_task_map:

tasks = _remap_inspect_tasks_for_merged_chains(tasks, edge_task_map)

if not tasks:

raise ReplanValidationError("No inspect segments in baseline mission")

cost_cfg = _weather_cost_config_for_dijkstra(weather_context)

visit_order, order_source = _compute_replan_edge_visit_order(

base_mission,

tasks,

topo_graph=topo_graph,

edge_task_map=edge_task_map or {},

start=start,

weather_context=weather_context,

cost_config=cost_cfg,

)

visit_order = [e for e in visit_order if e in tasks]

if not visit_order:

raise ReplanValidationError(

"No edges with inspection tasks after pruning empty branches"

)

visit_order, first_forward, first_entry = _rotate_order_nearest_start(

visit_order, tasks, start, weather_context=weather_context

)

topo_reload_ok = topo_graph is not None and bool(edge_task_map)

missing_topo_edges = [eid for eid in visit_order if eid not in edge_task_map]

if topo_reload_ok and missing_topo_edges:

segments_out: List[Dict[str, Any]] = []

directions: Dict[str, bool] = {}

seg_idx = 0

current: Point = start

prev_edge: Optional[str] = None

connect_start_geom = _connect_geometry_topo(

start,

first_entry,

role="from_start",

from_edge_id=None,

to_edge_id=visit_order[0],

topo_graph=topo_graph,

edge_task_map=edge_task_map,

cost_config=cost_cfg,

)

_cs_len = _path_length(connect_start_geom)

_cs_n = len(connect_start_geom)

_cs_f = _fmt_xy_pt(connect_start_geom[0]) if _cs_n else "(?,?)"

_cs_l = _fmt_xy_pt(connect_start_geom[-1]) if _cs_n else "(?,?)"

segments_out.append(

_make_connect_segment(

seg_idx,

connect_start_geom,

role="from_start",

from_edge_id=None,

to_edge_id=visit_order[0],

segment_id="connect_from_start",

)

)

seg_idx += 1

current = first_entry

for i, eid in enumerate(visit_order):

poly = tasks[eid]["polyline"]

if i == 0:

forward = first_forward

else:

d_fwd = _dist(current, poly[0]) + _weather_penalty_for_line(current, poly[0], weather_context)

d_rev = _dist(current, poly[-1]) + _weather_penalty_for_line(current, poly[-1], weather_context)

forward = d_fwd <= d_rev

directions[eid] = forward

inspect_geom = poly if forward else list(reversed(poly))

entry: Point = inspect_geom[0]

exit_pt: Point = inspect_geom[-1]

if i > 0 and _dist(current, entry) > 0.5:

connect_geom = _connect_geometry_topo(

current,

entry,

role="between_edges",

from_edge_id=prev_edge,

to_edge_id=eid,

topo_graph=topo_graph,

edge_task_map=edge_task_map,

cost_config=cost_cfg,

)

_cs_len = _path_length(connect_geom)

_cs_n = len(connect_geom)

_cs_f = _fmt_xy_pt(connect_geom[0]) if _cs_n else "(?,?)"

_cs_l = _fmt_xy_pt(connect_geom[-1]) if _cs_n else "(?,?)"

segments_out.append(

_make_connect_segment(

seg_idx,

connect_geom,

role="between_edges",

from_edge_id=prev_edge,

to_edge_id=eid,

)

)

seg_idx += 1

current = entry

segments_out.append(

_make_inspect_segment(

seg_idx,

inspect_geom,

eid,

from_edge_id=prev_edge,

direction="forward" if forward else "reverse",

)

)

seg_idx += 1

current = exit_pt

prev_edge = eid

connect_end_geom = _connect_geometry_topo(

current,

end,

role="to_end",

from_edge_id=prev_edge,

to_edge_id=None,

topo_graph=topo_graph,

edge_task_map=edge_task_map,

cost_config=cost_cfg,

)

end_connected = _dist(connect_end_geom[-1], end) < 2.0

_ce_len = _path_length(connect_end_geom)

_ce_n = len(connect_end_geom)

_ce_f = _fmt_xy_pt(connect_end_geom[0]) if _ce_n else "(?,?)"

_ce_l = _fmt_xy_pt(connect_end_geom[-1]) if _ce_n else "(?,?)"

segments_out.append(

_make_connect_segment(

seg_idx,

connect_end_geom,

role="to_end",

from_edge_id=prev_edge,

to_edge_id=None,

segment_id="connect_to_end",

)

)

seg_idx += 1

if not end_connected:

raise ReplanValidationError(

"Failed to connect final path to end point"

)

inspect_len = sum(

s["length"] for s in segments_out if s["type"] == "inspect"

)

connect_len = sum(

s["length"] for s in segments_out if s["type"] == "connect"

)

total_len = inspect_len + connect_len

base_meta = copy.deepcopy(base_mission.get("metadata") or {})

ctx = mission_context or {}

planning_points, resolved_source = _resolve_replan_inspection_points(

base_mission,

visit_order=visit_order,

tasks=tasks,

directions=directions,

spacing=spacing,

mission_context=ctx,

)

if is_image_inspection_source(resolved_source):

resolved_source = "image"

# 底图路径以 baseline 为准，禁止用 planning ctx 覆盖为 test.png 等默认图

if ctx.get("image_detection_stats") and not base_meta.get("image_detection_stats"):

base_meta["image_detection_stats"] = copy.deepcopy(ctx.get("image_detection_stats"))

ovr = ctx.get("image_inspection_overlay")

if ovr and not (base_meta.get("image_inspection_overlay") or []):

base_meta["image_inspection_overlay"] = copy.deepcopy(ovr)

else:

resolved_source = "spacing"

base_meta["inspection_point_source"] = resolved_source

base_stats = copy.deepcopy(base_mission.get("statistics") or {})

statistics = {

**base_stats,

"total_length": round(total_len, 2),

"inspect_length": round(inspect_len, 2),

"connect_length": round(connect_len, 2),

"inspect_ratio": round(

(inspect_len / total_len * 100.0) if total_len > 0 else 0.0, 2

),

"num_segments": len(segments_out),

"num_inspection_points": len(planning_points),

"num_edges": len(visit_order),

}

mission = {

"metadata": {

**base_meta,

"planner_name": "StartEndReplanPlanner",

"replan_engine": "planner/replan_start_end.py",

},

"statistics": statistics,

"groups": copy.deepcopy(base_mission.get("groups") or []),

"visit_order": {

"edge_visit_order": visit_order,

"edge_direction": {

eid: ("forward" if directions[eid] else "reverse")

for eid in visit_order

},

"group_visit_order": base_mission.get("visit_order", {}).get(

"group_visit_order", []

)

if isinstance(base_mission.get("visit_order"), dict)

else [],

},

"segments": segments_out,

"inspection_points": planning_points,

"markers": {

"start": {"x": start[0], "y": start[1]},

"end": {"x": end[0], "y": end[1]},

},

"replan_metadata": {

"planner": "start_end_replan",

"start": [start[0], start[1]],

"end": [end[0], end[1]],

"end_connected": end_connected,

"planning_spacing": spacing,

"planning_point_count": len(planning_points),

"inspection_point_source": resolved_source,

"weather_aware": bool((weather_context or {}).get("enabled")),

"weather_weight": float((weather_context or {}).get("weather_weight", 1.0)),

"connect_planner": "dijkstra",

"topo_context_reload": topo_reload_ok,

"topo_reload_image": image_rel,

"topo_reload_source": src_for_topo,

"topo_missing_edges": missing_topo_edges[:32],

},

}

if is_image_inspection_source(resolved_source):

mission["image_inspection_overlay"] = copy.deepcopy(

ctx.get("image_inspection_overlay")

or base_meta.get("image_inspection_overlay")

or base_mission.get("image_inspection_overlay")

or []

)

return mission

# backend/05_dashboard_export.py

"""

GridVision 毕设答辩核心代码摘录 — Dashboard 导出

源文件: planner/mission_result_builder.py

行号: 292-360, 839-950

CV 坐标抽取 + build_dashboard_from_mission_json。

本文件为摘录，非可独立运行模块；完整依赖见原工程。

"""

# ===== mission_result_builder.py L292-L360 =====

def extract_cv_dashboard_xy_and_snapped(

pt: Any,

) -> Tuple[Optional[Tuple[float, float]], Optional[Tuple[float, float]]]:

"""

Dashboard 主坐标 x/y：优先 CV / 原始像素，不得用 snapped_coord 作为主来源。

顺序：original_pixel_x/y → raw_x/y → detection_result.raw_coord → pixel_position / position_2d / position。

吸附/投影坐标单独返回供 snapped_x / snapped_y / metadata。

"""

dr = _detection_dict(pt)

display: Optional[Tuple[float, float]] = None

if isinstance(pt, dict):

ox = _finite_float(pt.get("original_pixel_x"))

oy = _finite_float(pt.get("original_pixel_y"))

if ox is not None and oy is not None:

display = (ox, oy)

if display is None:

rx = _finite_float(pt.get("raw_x"))

ry = _finite_float(pt.get("raw_y"))

if rx is not None and ry is not None:

display = (rx, ry)

if display is None:

display = _xy_from_sequence(dr.get("raw_coord"))

if display is None and isinstance(pt, dict):

display = _xy_from_sequence(pt.get("raw_coord"))

if display is None:

display = _xy_from_sequence(

pt.get("pixel_position")

or pt.get("position_2d")

or pt.get("position")

or pt.get("pos2d")

)

else:

ox = _finite_float(getattr(pt, "original_pixel_x", None))

oy = _finite_float(getattr(pt, "original_pixel_y", None))

if ox is not None and oy is not None:

display = (ox, oy)

if display is None:

rx = _finite_float(getattr(pt, "raw_x", None))

ry = _finite_float(getattr(pt, "raw_y", None))

if rx is not None and ry is not None:

display = (rx, ry)

if display is None:

display = _xy_from_sequence(dr.get("raw_coord"))

if display is None:

display = _xy_from_sequence(getattr(pt, "raw_coord", None))

if display is None:

pos = getattr(pt, "pixel_position", None) or getattr(pt, "pos2d", None) or getattr(pt, "position", None)

display = _xy_from_sequence(pos)

snapped: Optional[Tuple[float, float]] = _xy_from_sequence(dr.get("snapped_coord"))

if snapped is None and isinstance(pt, dict):

snapped = _xy_from_sequence(pt.get("snapped_coord"))

if snapped is None and isinstance(pt, dict):

sx = _finite_float(pt.get("snapped_x"))

sy = _finite_float(pt.get("snapped_y"))

if sx is not None and sy is not None:

snapped = (sx, sy)

if snapped is None and not isinstance(pt, dict):

snapped = _xy_from_sequence(getattr(pt, "snapped_coord", None))

if snapped is None and not isinstance(pt, dict):

sx = _finite_float(getattr(pt, "snapped_x", None))

sy = _finite_float(getattr(pt, "snapped_y", None))

if sx is not None and sy is not None:

snapped = (sx, sy)

return display, snapped

# ===== mission_result_builder.py L839-L950 =====

def build_dashboard_from_mission_json(

data: Dict[str, Any],

*,

pipeline: str,

source: str,

planner: str = "legacy",

input_file: str = "",

spacing: float = 50.0,

map_background: Optional[Dict[str, Any]] = None,

coordinate_mode: str = "auto_fit",

output_files: Optional[Dict[str, str]] = None,

extra_metadata: Optional[Dict[str, Any]] = None,

root: Optional[Path] = None,

) -> Dict[str, Any]:

"""从已导出的 mission_output.json 构建 Dashboard 结构。"""

segments = _segments_from_json(data)

image_catalog = _scan_inspection_image_catalog(root) if root else []

inspection_points = _inspection_points_from_json(

data,

segments,

root=root,

image_catalog=image_catalog,

)

statistics = _statistics_from_json(data, inspection_points)

visit = data.get("visit_order", {})

visit_order = visit.get("edge_visit_order", []) if isinstance(visit, dict) else (visit or [])

metadata: Dict[str, Any] = {

"pipeline": pipeline,

"source": source,

"planner": planner,

"input_file": input_file,

"spacing": spacing,

"coordinate_mode": coordinate_mode,

}

if extra_metadata:

metadata.update(extra_metadata)

if pipeline == "image":

bg_src = (

metadata.get("clean_map_image")

or metadata.get("display_map_image")

or metadata.get("map_image")

or input_file

or "data/test.png"

)

metadata["background"] = str(bg_src).replace("\\", "/")

merge_mission_metadata_into_dashboard(metadata, data)

payload: Dict[str, Any] = {

"segments": segments,

"inspection_points": inspection_points,

"statistics": statistics,

"visit_order": visit_order,

"group_visit_order": visit.get("group_visit_order", []) if isinstance(visit, dict) else [],

"markers": _start_end_from_segments(segments),

"metadata": metadata,

"output_files": output_files or {},

"map_background": map_background,

"map_mode": "image_overlay" if pipeline == "image" else "topology_only",

"image_inspection_overlay": (

data.get("image_inspection_overlay")

or metadata.get("image_inspection_overlay")

or []

),

}

pixel_fixed = bool(

metadata.get("pixel_coordinate_mode")

or metadata.get("coordinate_mode") == "image_pixel_fixed"

or str(metadata.get("coordinate_mode") or "").startswith("image_pixel")

)

if pixel_fixed:

coordinate_mode = "image_fixed"

metadata["coordinate_mode"] = "image_pixel_fixed"

iw = metadata.get("image_width")

ih = metadata.get("image_height")

if (not iw or not ih) and map_background:

iw = map_background.get("width")

ih = map_background.get("height")

if iw and ih:

payload["bounds"] = {

"x_range": [0, float(iw)],

"y_range": [float(ih), 0],

"width": float(iw),

"height": float(ih),

}

elif map_background and map_background.get("axis"):

payload["bounds"] = {

"x_range": map_background["axis"]["x_range"],

"y_range": map_background["axis"]["y_range"],

"width": map_background.get("width"),

"height": map_background.get("height"),

}

elif coordinate_mode == "auto_fit":

payload["bounds"] = compute_geometry_bounds(segments, inspection_points)

elif map_background and map_background.get("axis"):

payload["bounds"] = {

"x_range": map_background["axis"]["x_range"],

"y_range": map_background["axis"]["y_range"],

"width": map_background.get("width"),

"height": map_background.get("height"),

}

_img_stats = _inspection_image_stats_dict(inspection_points, root)

metadata["inspection_image_stats"] = _img_stats

return payload

# frontend/01_map_render.js

/**

* GridVision 毕设答辩核心代码摘录 — 地图路径可视化

* 源文件: web/static/app.js

* 行号: 1505-1710, 1898-2250, 2307-2531

* 说明: 摘录供答辩展示，需与原工程 static 资源配合运行。

*/

/* ===== app.js L1505-L1710 ===== */

const BAD_SNAPPED_DIST_PX = 40;

function inspectionPointCoordPair(a, b) {

const x = Number(a);

const y = Number(b);

return Number.isFinite(x) && Number.isFinite(y) ? { x, y } : null;

}

function inspectionPointRawXY(p) {

if (!p || typeof p !== "object") return null;

return (

inspectionPointCoordPair(p.raw_x, p.raw_y) ||

inspectionPointCoordPair(p.original_pixel_x, p.original_pixel_y) ||

inspectionPointCoordPair(p.x, p.y)

);

}

function isSnappedCoordUsable(p) {

if (!p || typeof p !== "object") return false;

const snapped = inspectionPointCoordPair(p.snapped_x, p.snapped_y);

if (!snapped) return false;

const raw = inspectionPointRawXY(p);

if (!raw) return true;

const dist = Math.hypot(raw.x - snapped.x, raw.y - snapped.y);

if (dist <= BAD_SNAPPED_DIST_PX) return true;

return false;

}

/** Dashboard 主图坐标：snapped 仅在 raw 偏移 ≤40px 时采用，否则 fallback */

function inspectionPointDisplayXY(p) {

if (!p || typeof p !== "object") return null;

if (isSnappedCoordUsable(p)) {

const snapped = inspectionPointCoordPair(p.snapped_x, p.snapped_y);

if (snapped) return snapped;

}

let r = inspectionPointCoordPair(p.display_x, p.display_y);

if (r) return r;

if (p.metadata && typeof p.metadata === "object") {

r = inspectionPointCoordPair(p.metadata.display_x, p.metadata.display_y);

if (r) return r;

}

r = inspectionPointRawXY(p);

if (r) return r;

const dr = p.detection_result || {};

const rc = dr.raw_coord || p.raw_coord;

if (Array.isArray(rc) && rc.length >= 2) {

r = inspectionPointCoordPair(rc[0], rc[1]);

if (r) return r;

}

const pos = p.pixel_position || p.position_2d || p.position || p.pos2d;

if (Array.isArray(pos) && pos.length >= 2) {

r = inspectionPointCoordPair(pos[0], pos[1]);

if (r) return r;

}

return inspectionPointCoordPair(p.x, p.y);

}

/**

* 用 mission.inspection_points 的 display x/y，在全部蓝色 inspect 段折线边上找最近投影；

* 确定性插入：每点投影到最近边，同边上按 t 排序；不创建绿色连线 trace。

* @param {Array<{ geom: number[][], hoverLine: string }>} inspectSegs

* @param {object} mission

*/

function augmentInspectSegmentsWithInspectionPoints(inspectSegs, mission) {

const drawPts = Array.isArray(mission?.inspection_points) ? mission.inspection_points : [];

const m = drawPts.length;

const inspectionRows = [];

const missingRows = [];

drawPts.forEach((p, idx) => {

const xy = inspectionPointDisplayXY(p);

const pointId = String(p.point_id || p.id || idx + 1).replace(/^point_/, "");

if (!xy) {

missingRows.push({

point_id: pointId,

x: p.x ?? "—",

y: p.y ?? "—",

nearest_dist: null,

});

return;

}

const totalPoints = m;

const pointLabel = `巡检点 ${pointId}`;

const segmentLabel = p.segment_id

? `巡检区段 ${String(p.segment_id).replace(/^seg_/, "")}`

: "—";

const cd = [

segmentLabel,

p.image_available ? "真实图片" : "占位图",

`${p.progress_index || idx + 1}/${totalPoints}`,

pointLabel,

];

inspectionRows.push({ x: xy.x, y: xy.y, cd, idx, point_id: pointId });

});

const auditAndReturn = (inspectX, inspectY, inspectCustom, inserted) => {

return { inspectX, inspectY, inspectCustom };

};

if (!inspectSegs?.length) {

drawPts.forEach((p, idx) => {

if (missingRows.some((r) => r.point_id === String(p.point_id || p.id || idx + 1).replace(/^point_/, ""))) {

return;

}

const xy = inspectionPointDisplayXY(p);

if (xy) {

missingRows.push({

point_id: String(p.point_id || p.id || idx + 1).replace(/^point_/, ""),

x: xy.x,

y: xy.y,

nearest_dist: null,

});

}

});

const flat = flattenInspectSegsToPlotly(inspectSegs || []);

return auditAndReturn(flat.xs, flat.ys, flat.customs, 0);

}

if (!inspectionRows.length) {

const flat = flattenInspectSegsToPlotly(inspectSegs);

return auditAndReturn(flat.xs, flat.ys, flat.customs, 0);

}

const edges = [];

let arcBase = 0;

inspectSegs.forEach((seg, segIdx) => {

const g = seg.geom;

for (let j = 0; j < g.length - 1; j += 1) {

const ax = g[j][0];

const ay = g[j][1];

const bx = g[j + 1][0];

const by = g[j + 1][1];

const dx = bx - ax;

const dy = by - ay;

const len = Math.hypot(dx, dy);

edges.push({ ax, ay, bx, by, dx, dy, len, arc0: arcBase, segIdx, localJ: j });

arcBase += len;

}

});

if (!edges.length) {

inspectionRows.forEach((row) => {

missingRows.push({

point_id: row.point_id,

x: row.x,

y: row.y,

nearest_dist: null,

});

});

const flat = flattenInspectSegsToPlotly(inspectSegs);

/* ===== app.js L1898-L2250 ===== */

function expandInspectSegmentByArc(seg, pointsOnSeg, segmentId) {

const geom = seg.geometry_2d || [];

const sid = segmentId || String(seg.segment_id || "");

const rawCount = geom.length;

const failedPoints = [];

const insertMeta = [];

if (geom.length < 2) {

(pointsOnSeg || []).forEach((pt) => {

failedPoints.push({ pt, reason: "empty_geometry" });

});

return { frames: [], insertMeta, failedPoints, rawCount, pointCount: (pointsOnSeg || []).length };

}

const vertexArcs = polylineVertexArcs(geom);

const items = geom.map((p, i) => ({

x: Number(p[0]),

y: Number(p[1]),

arc: vertexArcs[i],

kind: "vertex",

point_ids: [],

orderIndex: -1,

}));

const insertedIds = new Set();

(pointsOnSeg || []).forEach((pt) => {

if (!Number.isFinite(pt.x) || !Number.isFinite(pt.y)) {

failedPoints.push({ pt, reason: "invalid_coords" });

return;

}

const vi = findVertexMatchIndex(pt, geom, POINT_FRAME_EPS);

if (vi >= 0) {

appendPointIdToItem(items[vi], pt.point_id);

insertedIds.add(pt.point_id);

insertMeta.push({

point_id: pt.point_id,

arc: vertexArcs[vi],

x: pt.x,

y: pt.y,

});

return;

}

const proj = projectPointOnPolyline(pt.x, pt.y, geom);

if (!proj || !Number.isFinite(proj.arcS)) {

failedPoints.push({ pt, reason: "no_projection" });

return;

}

items.push({

x: pt.x,

y: pt.y,

arc: proj.arcS,

t: proj.t,

kind: "inspection",

point_ids: [pt.point_id],

orderIndex: pt.orderIndex != null ? pt.orderIndex : 0,

});

insertedIds.add(pt.point_id);

insertMeta.push({

point_id: pt.point_id,

arc: proj.arcS,

x: pt.x,

y: pt.y,

});

});

(pointsOnSeg || []).forEach((pt) => {

if (!insertedIds.has(pt.point_id)) {

if (!failedPoints.some((f) => f.pt.point_id === pt.point_id)) {

failedPoints.push({ pt, reason: "not_inserted" });

}

}

});

items.sort((a, b) => {

if (Math.abs(a.arc - b.arc) > 1e-9) return a.arc - b.arc;

if (a.kind !== b.kind) return a.kind === "vertex" ? -1 : 1;

if (a.kind === "inspection" && b.kind === "inspection") {

return a.orderIndex - b.orderIndex;

}

return 0;

});

const deduped = [];

items.forEach((it) => {

const prev = deduped[deduped.length - 1];

if (

prev &&

Math.abs(prev.x - it.x) <= ROUTE_SEQ_EPS &&

Math.abs(prev.y - it.y) <= ROUTE_SEQ_EPS

) {

prev.point_ids = mergeRoutePointIds(prev.point_ids, it.point_ids);

if (it.kind === "inspection") {

prev.kind = "inspection";

}

return;

}

deduped.push({

...it,

point_ids: normalizeRoutePointIds(it.point_ids),

});

});

const frames = deduped.map((it) => ({

x: it.x,

y: it.y,

...frameRouteFields(it.kind === "inspection" ? it.point_ids : []),

}));

const pointCount = (pointsOnSeg || []).length;

const finalCount = frames.length;raw=${rawCount} points=${pointCount} final=${finalCount} first=(${frames[0].x},${frames[0].y}) last=(${frames[finalCount - 1].x},${frames[finalCount - 1].y})`

);

}

return {

frames,

insertMeta,

failedPoints,

rawCount,

pointCount,

finalCount,

};

}

function buildMissionRouteSequence(mission, rowsWithXY) {

const markers = mission?.markers || {};

const meta = mission?.metadata || {};

const sx = Number(markers.start?.x ?? meta.start?.x ?? meta.start_point?.x);

const sy = Number(markers.start?.y ?? meta.start?.y ?? meta.start_point?.y);

const segments = mission?.segments || [];

const dist2 = (a, b) => Math.hypot(a.x - b.x, a.y - b.y);

const inspectPoints = [];

(rowsWithXY || []).forEach((r, i) => {

const p = r.point || {};

const pointId = String(p.point_id || p.id || i + 1).trim();

let boundSeg = String(p.segment_id || "").trim();

if (!boundSeg) {

for (const seg of segments) {

if (isSkippableMissionDebugSegment(seg) || seg.type !== "inspect") continue;

const geom = seg.geometry_2d || [];

if (geom.length < 2) continue;

if (pointToPolylineDist(r.x, r.y, geom) <= ON_GEOM_EPS) {

boundSeg = String(seg.segment_id || "");

break;

}

}

}

if (!boundSeg) {

let bestD = Infinity;

for (const seg of segments) {

if (isSkippableMissionDebugSegment(seg) || seg.type !== "inspect") continue;

const d = pointToPolylineDist(r.x, r.y, seg.geometry_2d || []);

if (d < bestD) {

bestD = d;

boundSeg = String(seg.segment_id || "");

}

}

}

inspectPoints.push({

point_id: pointId,

id: pointId,

x: r.x,

y: r.y,

image_url:

(typeof p.image_url === "string" && p.image_url.trim()) ||

`/api/inspection-image/${pointId}.jpg`,

segment_id: boundSeg,

edge_id: p.edge_id || "",

orderIndex: i,

});

});

const pointsBySegment = new Map();

inspectPoints.forEach((pt) => {

const sid = pt.segment_id;

if (!sid) return;

if (!pointsBySegment.has(sid)) pointsBySegment.set(sid, []);

pointsBySegment.get(sid).push(pt);

});

const sequence = [];

let skippedStartDup = false;

const pushFrame = (frame) => {

const prev = sequence[sequence.length - 1];

const incomingIds = frame.routePointIds || (frame.routePointId ? [frame.routePointId] : []);

if (prev && dist2(prev, frame) <= ROUTE_SEQ_EPS) {

const merged = mergeRoutePointIds(prev.routePointIds, incomingIds);

prev.routePointIds = merged;

prev.routePointId = merged.length === 1 ? merged[0] : null;

return;

}

sequence.push({

...frame,

...frameRouteFields(incomingIds),

});

};

if (Number.isFinite(sx) && Number.isFinite(sy)) {

pushFrame({

segment_id: "start",

type: "start",

segment_type: "start",

x: sx,

y: sy,

edge_id: null,

routePointIds: [],

routePointId: null,

});

}

segments.forEach((seg) => {

if (isSkippableMissionDebugSegment(seg)) return;

const geom = seg.geometry_2d || [];

if (geom.length < 2) return;

const sid = String(seg.segment_id || "");

const stype =

seg.type === "inspect"

? "inspect"

: seg.type === "connect"

? "connect"

: String(seg.type || "move");

const edgeId = seg.edge_id != null ? String(seg.edge_id) : null;

if (stype === "inspect") {

const segStartIdx = sequence.length;

const expanded = expandInspectSegmentByArc(

seg,

pointsBySegment.get(sid) || [],

sid

);

expanded.frames.forEach((v, idx) => {

if (!skippedStartDup && idx === 0 && sequence.length) {

const startFrame = sequence[0];

if (

startFrame?.type === "start" &&

dist2(startFrame, v) <= ROUTE_SEQ_EPS

) {

skippedStartDup = true;

return;

}

}

pushFrame({

segment_id: sid,

type: stype,

segment_type: stype,

x: v.x,

y: v.y,

edge_id: edgeId,

routePointIds: v.routePointIds || (v.routePointId ? [v.routePointId] : []),

routePointId: v.routePointId || null,

});

});

expanded.insertMeta.forEach((meta) => {

let frameIdx = -1;

for (let i = segStartIdx; i < sequence.length; i += 1) {

const ids = sequence[i].routePointIds || [];

if (ids.includes(meta.point_id)) {

frameIdx = i;

break;

}

}

if (frameIdx < 0) return;

});

return;

}

geom.forEach((p, idx) => {

const x = Number(p[0]);

const y = Number(p[1]);

if (!Number.isFinite(x) || !Number.isFinite(y)) return;

if (!skippedStartDup && idx === 0 && sequence.length) {

const startFrame = sequence[0];

if (

startFrame?.type === "start" &&

dist2(startFrame, { x, y }) <= ROUTE_SEQ_EPS

) {

skippedStartDup = true;

return;

}

}

pushFrame({

segment_id: sid,

type: stype,

segment_type: stype,

x,

y,

edge_id: edgeId,

routePointIds: [],

routePointId: null,

});

});

});

const pointFrameIndex = {};

sequence.forEach((f, i) => {

(f.routePointIds || []).forEach((pid) => {

pointFrameIndex[pid] = i;

});

});

inspectPoints.forEach((pt) => {

pt.trigger_frame = pointFrameIndex[pt.point_id] ?? -1;

});  sequence.forEach((f, i) => {

const ids = f.routePointIds || [];

if (ids.length > 1) {

}

});

window.__MISSION_ROUTE_SEQUENCE = sequence;

window.__INSPECTION_POINT_BINDINGS = inspectPoints;

const segSummary = [];

let lastKey = null;

sequence.forEach((f) => {

const key = `${f.segment_id}:${f.segment_type || f.type}`;

if (key !== lastKey) {

segSummary.push(key);

lastKey = key;

}

});

/* ===== app.js L2307-L2531 ===== */

function buildMissionTraces(missionPayload) {

const mission = missionPayload || getCurrentMission();

const inspectionPointsLengthAtEntry = mission?.inspection_points?.length ?? 0;

const segments = mission?.segments || [];

const inspectSegs = [];

const connectX = [],

connectY = [],

connectCustom = [];

segments.forEach((seg) => {

if (isSkippableMissionDebugSegment(seg)) return;

const geom = seg.geometry_2d || [];

if (geom.length < 2) return;

const segLabel = seg.segment_id

? `巡检区段 ${String(seg.segment_id).replace(/^seg_/, "")}`

: "巡检区段";

const typeLabel = seg.type === "inspect" ? "巡检段" : seg.type === "connect" ? "连接段" : "区段";

const hoverLine = `${segLabel}<br>${typeLabel} · ${Math.round(seg.length || 0)}px`;

if (seg.type === "inspect") {

inspectSegs.push({

geom: geom.map((p) => [Number(p[0]), Number(p[1])]),

hoverLine,

});

} else if (seg.type === "connect") {

connectX.push(...geom.map((p) => p[0]), null);

connectY.push(...geom.map((p) => p[1]), null);

connectCustom.push(...geom.map(() => hoverLine), null);

}

});

[connectX, connectY, connectCustom].forEach((arr) => {

if (arr.length && arr[arr.length - 1] === null) arr.pop();

});

/** Dashboard：巡检点 marker（不连线）；蓝线顶点 = 原 inspect 折线 + 按弧长投影插入的巡检点 */

const drawPts = Array.isArray(mission.inspection_points) ? mission.inspection_points : [];

const rowsWithXY = [];

if (drawPts.length) {

const totalPoints = drawPts.length;

drawPts.forEach((p, idx) => {

const xy = inspectionPointDisplayXY(p);

if (!xy) return;

const pointLabel = `巡检点 ${String(p.point_id || p.id || idx + 1).replace(/^point_/, "")}`;

const segmentLabel = p.segment_id

? `巡检区段 ${String(p.segment_id).replace(/^seg_/, "")}`

: "—";

const cd = [

segmentLabel,

p.image_available ? "真实图片" : "占位图",

`${p.progress_index || idx + 1}/${totalPoints}`,

pointLabel,

];

rowsWithXY.push({ x: xy.x, y: xy.y, cd, missionIndex: idx, point: p });

});

}

let inspectX = [];

let inspectY = [];

let inspectCustom = [];

if (inspectSegs.length) {

const aug = augmentInspectSegmentsWithInspectionPoints(inspectSegs, mission);

inspectX = aug.inspectX;

inspectY = aug.inspectY;

inspectCustom = aug.inspectCustom;

} else if (rowsWithXY.length) {

}

const traces = [];

const { trace: unvisitedRedTrace, diag: redDiag } = buildUnvisitedRedLinesTrace(mission);

traces.push(unvisitedRedTrace);

if (state.showInspect && inspectX.length) {

traces.push({

x: inspectX,

y: inspectY,

mode: "lines",

name: "巡检路径",

line: { color: MAP_STYLES.inspectColor, width: 4 },

hoverinfo: "text",

hovertext: inspectCustom,

showlegend: true,

legendgroup: "inspect",

legendrank: 1,

});

}

if (state.showConnect && connectX.length) {

traces.push({

x: connectX,

y: connectY,

mode: "lines",

name: "连接路径",

line: { color: MAP_STYLES.connectColor, width: 3, dash: "dash" },

hoverinfo: "text",

hovertext: connectCustom,

showlegend: true,

legendgroup: "connect",

legendrank: 2,

});

}

if (rowsWithXY.length) {

traces.push({

x: rowsWithXY.map((r) => r.x),

y: rowsWithXY.map((r) => r.y),

mode: "markers",

name: "巡检点",

marker: {

color: "#22c55e",

size: 11,

symbol: "circle",

line: { color: "#052e16", width: 1.5 },

},

text: rowsWithXY.map((r) => r.cd[3]),

customdata: rowsWithXY.map((r) => {

const p = r.point;

const pointId = String(p.point_id || p.id || "").trim();

return {

point_id: pointId,

image_url:

(typeof p.image_url === "string" && p.image_url.trim()) ||

`/api/inspection-image/${pointId}.jpg`,

};

}),

hovertemplate:

"<b>%{text}</b><br>" +

"point_id: %{customdata.point_id}<br>" +

"url: %{customdata.image_url}<br>" +

"<span style='opacity:0.85'>点击在侧栏查看巡检图</span><extra></extra>",

hoverinfo: "text",

showlegend: true,

legendgroup: "points",

legendrank: 3,

});

} else {

}

const markers = mission.markers || {};

if (markers.start) {

traces.push({

x: [markers.start.x],

y: [markers.start.y],

mode: "markers+text",

name: "起点",

marker: {

size: 16,

color: "#22c55e",

symbol: "star",

line: { width: 2, color: "#14532d" },

},

text: ["起点"],

textfont: { size: 10, color: "#bbf7d0" },

textposition: "top center",

hovertemplate: "起点 (%{x:.0f}, %{y:.0f})<extra></extra>",

showlegend: true,

legendgroup: "start",

legendrank: 7,

});

}

if (markers.end) {

traces.push({

x: [markers.end.x],

y: [markers.end.y],

mode: "markers+text",

name: "终点",

marker: {

size: 16,

color: "#ef4444",

symbol: "diamond",

line: { width: 2, color: "#7f1d1d" },

},

text: ["终点"],

textfont: { size: 10, color: "#fecaca" },

textposition: "bottom center",

hovertemplate: "终点 (%{x:.0f}, %{y:.0f})<extra></extra>",

showlegend: true,

legendgroup: "end",

legendrank: 8,

});

}

cacheDrawnRouteForPlayback(mission, connectX, connectY, inspectX, inspectY, rowsWithXY);

buildMissionRouteSequence(mission, rowsWithXY);

return traces;

}

# frontend/02_playback.js

/**

* GridVision 毕设答辩核心代码摘录 — 巡检播放

* 源文件: web/static/playback.js

* 行号: 1345-1434, 1615-1680

* 说明: 摘录供答辩展示，需与原工程 static 资源配合运行。

*/

/* ===== playback.js L1345-L1434 ===== */

function tickPlayback() {

if (PLAYBACK.status !== "playing") return;

if (PLAYBACK.index >= PLAYBACK.timeline.length) {

PLAYBACK.status = "finished";

PLAYBACK.running = false;

PLAYBACK.paused = false;

notifyPlaybackPhase("finished");

stopTimer();

stopStreamLoop();

PLAYBACK.holdInspectSnapshot = false;

PLAYBACK.streamFrozen = false;

markAllInspectionPointsVisited();      setMissionState("COMPLETED");

updateTelemetryFromPlayback("COMPLETED");

pushEvent("任务执行完成");

setPlaybackUi();

if (ENABLE_INSPECTION_IMAGE) {

["", "Fs"].forEach((p) => {

const st = $(`inspectCardStatus${p}`);

if (st) {

st.textContent = "任务完成";

st.className = "inspect-status done";

}

});

}

updateAllPlotPlayback();

return;

}

const ev = PLAYBACK.timeline[PLAYBACK.index];

const frameIndex = PLAYBACK.index;

PLAYBACK.index += 1;

PLAYBACK.uavPos = { x: ev.x, y: ev.y };

PLAYBACK.currentRouteIndex = ev.routeIndex != null ? ev.routeIndex : frameIndex;

PLAYBACK.currentPoint = null;

PLAYBACK.currentPointId = null;

checkInspectTriggersAtFrame(ev, frameIndex);segment=${ev.segment_id || ""} point=${tickPointIds.join(",")}`

);

}

setMissionState("EXECUTING");

updateTelemetryFromPlayback("EXECUTING");

updateAllPlotPlayback();

setPlaybackUi();

scheduleNext(MOVE_FRAME_MS / getSpeed());

}

function getFramePointIds(frame) {

if (Array.isArray(frame.routePointIds)) return frame.routePointIds;

if (frame.routePointId) return [frame.routePointId];

return [];

}

function triggerInspectionPoint(pointId) {

if (!pointId || PLAYBACK.visitedIds.has(pointId)) return;

const points = PLAYBACK.inspectPlotPoints || [];

const pt = points.find((p) => (p.point_id || p.id) === pointId);

if (!pt) return;

PLAYBACK.visitedIds.add(pointId);

PLAYBACK.currentPointId = pointId;

PLAYBACK.currentPoint = pt;

if (typeof window.showInspectionImage === "function") {

window.showInspectionImage(pt, { logAs: "route-point" });

}

if (ENABLE_INSPECTION_IMAGE) {

const total = PLAYBACK.totalInspectCount || points.length;

showInspectCard(

pt,

pt.orderIndex >= 0 ? pt.orderIndex + 1 : PLAYBACK.visitedIds.size,

total,

{ skipInspectionImage: true }

);

}

}

function checkInspectTriggersAtFrame(ev, frameIndex) {

const ids = getFramePointIds(ev);

ids.forEach((pointId) => {

triggerInspectionPoint(pointId);

});

}

function getPlotlyMapElement() {

/* ===== playback.js L1615-L1680 ===== */

window.startInspectionPlayback = function () {

if (PLAYBACK.status === "playing" || PLAYBACK._startLock) return;

PLAYBACK._startLock = true;

stopTimer();

stopStreamLoop();

clearPhaseTimers();

PLAYBACK.timeline = [];

PLAYBACK.index = 0;

PLAYBACK.t = 0;

PLAYBACK.running = false;

PLAYBACK.paused = false;

PLAYBACK.inspectPlotPoints = [];

const result = getMissionForPlayback();

if (!result) {

PLAYBACK._startLock = false;

return;

}

const timeline = buildTimelineFromMissionRouteSequence();

if (!timeline || !timeline.length) {

PLAYBACK._startLock = false;

return;

}

PLAYBACK.timeline = timeline;

PLAYBACK.inspectPlotPoints =

window.__INSPECTION_POINT_BINDINGS ||

window.__DRAWN_ROUTE_DEBUG?.inspect_points ||

[];

PLAYBACK.running = true;

PLAYBACK.paused = false;

PLAYBACK.status = "playing";

PLAYBACK.speed = getSpeed();

PLAYBACK.dwellMs = getDwellMs();

PLAYBACK.startTimestamp = Date.now();

PLAYBACK.streamFrozen = false;

PLAYBACK.holdInspectSnapshot = false;

PLAYBACK.totalInspectCount = PLAYBACK.inspectPlotPoints.length;

PLAYBACK.visitedIds = new Set();

PLAYBACK.visitedCoords = new Map();

PLAYBACK.activatedInspectSegments = new Set();

PLAYBACK.uavPos = { x: timeline[0].x, y: timeline[0].y };

PLAYBACK.currentPoint = null;

PLAYBACK.currentPointId = null;

notifyPlaybackPhase("playing");

setMissionState("EXECUTING");

clearInspectCard();

pushEvent("任务开始执行");

updateTelemetryFromPlayback("EXECUTING");

setPlaybackUi();

updateAllPlotPlayback();

tickPlayback();

PLAYBACK._startLock = false;

};

window.pauseInspectionPlayback = function () {

if (PLAYBACK.status !== "playing") return;

stopTimer();

stopStreamLoop();

clearPhaseTimers();

PLAYBACK.status = "paused";

# frontend/03_image_stream.js

/**

* GridVision 毕设答辩核心代码摘录 — 图传切换

* 源文件: web/static/playback.js

* 行号: 496-551

* 说明: 摘录供答辩展示，需与原工程 static 资源配合运行。

*/

/* ===== playback.js L496-L551 ===== */

function buildStreamPool(result) {

const set = new Set();

(result?.inspection_points || []).forEach((pt) => {

[pt.image_url, pt.image_path, pt.image_placeholder].forEach((v) => {

if (v && typeof v === "string") set.add(v);

});

});

for (let i = 1; i <= 36; i += 1) {

const name = `IP_${String(i).padStart(4, "0")}.jpg`;

set.add(`/api/inspection-image/${name}`);

}

STREAM_FALLBACKS.forEach((v) => set.add(v));

return [...set];

}

function stopStreamLoop() {

if (PLAYBACK.streamTimer != null) {

clearTimeout(PLAYBACK.streamTimer);

PLAYBACK.streamTimer = null;

}

}

function clearPhaseTimers() {

PLAYBACK.phaseTimers.forEach((t) => clearTimeout(t));

PLAYBACK.phaseTimers = [];

}

function streamDelayByState() {

if (PLAYBACK.missionState === "APPROACHING") {

return Math.round(700 + Math.random() * 250);

}

if (PLAYBACK.missionState === "TAKEOFF") {

return Math.round(460 + Math.random() * 260);

}

return Math.round(STREAM_INTERVAL_MIN + Math.random() * (STREAM_INTERVAL_MAX - STREAM_INTERVAL_MIN) * 0.6);

}

function scheduleStreamLoop() {

if (!ENABLE_INSPECTION_IMAGE) return;

stopStreamLoop();

if (PLAYBACK.status !== "playing" || PLAYBACK.streamFrozen) return;

PLAYBACK.streamTimer = setTimeout(() => {

if (PLAYBACK.status !== "playing" || PLAYBACK.streamFrozen) return;

const state = PLAYBACK.missionState;

const streamPool =

state === "APPROACHING" ? APPROACH_STREAM_IMAGES : CRUISE_STREAM_IMAGES;

PLAYBACK.streamIndex = (PLAYBACK.streamIndex + 1) % streamPool.length;

const hintText =

state === "APPROACHING"

? "接近巡检点 · 图传轻微闪烁"

: "实时图传 · 巡航占位画面";

setStreamImage(streamPool[PLAYBACK.streamIndex], hintText);

updateTelemetryFromPlayback(PLAYBACK.missionState);

scheduleStreamLoop();

}, streamDelayByState());

}